תאריך עדכון: 20.11.2023

סימוכין: 133-16

מאשר: אבי חיים, ראש אגף קרקעות מזוהמות, שפכי תעשייה ודלקים

גרסה: 2 (מחליפה גרסה 1 מתאריך 21.04.2016)

# נספח א' – רשימת שיטות דיגום ואנליזה

נספח זה הינו חלק מהנחיות מקצועיות לביצוע סקר קרקע

## תוכן עניינים

טבלאות

- טבלה 1 – פירוט כלל שיטות דיגום קרקע ואנליזה........................................................... 3

- טבלה 2 – פירוט בקרות איכות הנדרשות בביצוע במהלך סקר קרקע................................. 8

רשימות סטנדרטיות לבדיקות אנליטיות של מזהמים

- רשימה 1 – נדיפים (VOCs).......................................................................................... 11

- רשימה 2 – גז-קרקע אקטיבי TO-15............................................................................. 13

רשימה 3 – חצי נדיפים (SVOCs).................................................................................. 15

- רשימה 4 – מתכות (Metals)......................................................................................... 16

- רשימה 5 – חומרי נפץ (Explosives).............................................................................. 17

שינוע דגימות לחו"ל

- הנחיות לשינוע דגימות קרקע לחו"ל.............................................................................. 18

## הקדמה

נספח זה מרכז את סוגי בדיקות הקרקע הנפוצות במהלך חקירת קרקע, בדיקות לאיכות ובקרה, שיטות אנליטיות נדרשות ושיטות הקדיחה ודיגום המקובלות לכל בדיקה. להלן מספר נושאים חשובים בעניין דוחות המעבדה וניתוח ממצאים:

- על מנת ליצור אחידות במפרט המזהמים הנבדקים במעבדות השונות והמדווחים בתעודות המעבדה, צורפו לנספח זה רשימות סטנדרטיות מעודכנות (Analytical List 1-5) לבדיקת מזהמים בכל שיטה (נדיפים, גז אקטיבי TO-15, חצי נדיפים, מתכות וחומרי נפץ). המעבדות האנליטיות נדרשות להיערך בהתאם ולבצע בעבודה שוטפת אנליזות על פי רשימת המזהמים המפורטים ברשימות לכל שיטה. מעבר לכך, במקרים בהם סקר ההיסטורי יצביע על התכנות להמצאות מזהמים שאינם מופיעים ברשימות הנ"ל, יש לציין בפני המעבדה האנליטית את סוג המזהם הנדרש.

- חשוב להבחין בין 3 המושגים הבאים המשפיעים על איכות הבדיקה, ניתוח תוצאות וקבלת החלטות להמשך טיפול באתר נחקר:

| מושג | אופן כתיבה בדוח המעבדה האנליטית |  |
| --- | --- | --- |
| סף גילוי | LOD – Limit of Detection MDL – Minimum Detection Limit | הריכוז המינימלי של מזהם שהמכשיר האנליטי מסוגל לאתר בדוגמה ברמת וודאות של 99%. |
| סף כימות | LOQ – Limit of Quantification PQL – Practical Quantification Limit | הריכוז המינימלי של מזהם שהמכשיר מסוגל לכמת בדוגמה. |
| סף דיווח | LOR – Limit of Report RL – Report Limit | הריכוז המינימלי של מזהם בדוגמה שהמעבדה יכולה להתחייב לגביו בדרגת וודאות ודיוק סבירים. משתנה בהתאם לפעולות מקדימות, לדוגמה דילול, לפני ביצוע האנליזה. |

- המעבדות נדרשות לציין בדו"ח המעבדה את סף הדיווח (Limit of Report – LOR) בפועל של המזהמים הנבדקים באנליזה. בנוסף, חובת היועצים/מגישי דו"ח ממצאים לוודא שסף הדיווח של המעבדה האנליטית למזהם/ים הנדרשים בכל שיטה נמוך מערך ה-VSL של אותם מזהם/ים.

## TABLE 1

Analytical methods

| Analytical Parameter | Homogenization(1) Allowed? | Allowable Sampling Methods(2) | Method(s) | Required Container Type | Required Preservation matrix(5) | Holding Time from sample collection until analysis | Additional Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VOCs-Soil sampling (including BTEX/MTBE) **See analytical list below (No.1) | No | Direct push, Hollow stem auger, Spiral auger with conditions in regulations | Analytical method: EPA 8260B Preparation method: EPA 5021 (low-concentration(3)) | One (1) 22-mL clear glass vial with PTFE-lined septum and crimp-cap or screw-cap + 100-250 ml glass jar with Teflon-lined cap | Vial: with 10 mL matrix modifying solution (4) Vial & Jar: Cool, 4°C±2 | 14 days to analysis | Need about 2-gram sample (or until line marked on vial by the analytical lab). Must be collected with core-sampler. Transfer sample to vial and jar within 2 minutes of exposure to air. Glass jars do not replace vials and can be used only for lab screening during the first 48 hours from sampling. |
|  |  |  | Analytical method: EPA 8260B Preparation method: EPA 5021 (High-concentration(3)) | two (2) 22-mL clear glass vials with PTFE-lined septum and crimp-cap or screw-cap + 100-250 ml glass jar with Teflon-lined cap | Vial 1: with 10ml of methanol + Vial 2: with 10 mL matrix modifying solution (4) Vials & Jar: Cool, 4°C±2 | 14 days to analysis | Need about 2-gram sample (or until line marked on vial by the analytical lab). Must be collected with core-sampler. Transfer sample to vial and jar within 2 minutes of exposure to air. Glass jar is not replacing the vials and can be used only for lab screening during the first 48 hours from sampling. |
| Soil Gas sampling **See analytical list below (No.2) | Not relevant | Post-Run Tubing System (PRT) or Drive Point prob | EPA TO-15 | canister 1L to 6L (minimum 1L) | no | Must arrive to analytical laboratory in 5 days. 30 days to analysis | Avoid hits and damages to canister during shipment. |
| SVOCs (including PAHs) **See analytical list below (No.3) | Yes | Direct push, Hollow stem auger, Bucket auger, Hand auger, Power auger, Spiral auger with conditions in regulations | EPA 8270C | 100-250 ml glass jar with Teflon-lined cap with no air space or Inert liner sealed on both ends | Cool, 4°C±2 | 14 days to extraction; 40 days from extraction to analysis | Standard measurement for SVOCs does not include PCBs and pesticides. |
| TPH-DRO and TPH-ORO (C10-C40) | No | Direct push, Hollow stem auger, Bucket auger, Hand auger, Spiral auger with conditions in regulations | EPA 8015 | 100-250 ml glass jar with Teflon-lined cap with no air space or Inert liner sealed on both ends | Cool, 4°C±2 | 14 days to extraction; 40 days from extraction to analysis |  |
| TPH-GRO (C6-C10) | No | Direct push, Hollow stem auger, Spiral auger with conditions in regulations | Analytical method: EPA 8015 Preparation method: EPA 5021 | 22-mL clear glass vial with PTFE-lined septum and crimp-cap or screw-caps + 100-250 ml glass jar with Teflon-lined cap | Vial: with 10 mL matrix modifying solution (4) Cool, 4°C±2 | 14 days to analysis | Need about 2-gram sample (or until line marked on vial by the analytical lab). Must be collected with core-sampler. Transfer sample to vial and jar within 2 minutes of exposure to air. Glass jar does not replace the vials and can be used only for lab screening during the first 48 hours from sampling. |
| Metals **See analytical list below (NO.4) | Yes | Direct push, Hollow stem auger, Bucket auger, Hand auger, Power auger, Spiral auger with conditions in regulations | Analytical method: EPA 6010D, 6020A Acid : Preparation method: 3051A /3050B Analytical method: EPA 6010D, 6020A Leaching: Preparation method: EN-12457-2 | 100-250 ml glass jar with Teflon-lined cap with no air space or Inert liner sealed on both ends or Polyethylene container | Cool, 4°C±2 | 28 days for mercury; 6 months for other metals | All sample containers must be demonstrated to be free of contamination at or below the reporting limit. Plastic and glass containers are both suitable. Must be consistent with EPA metal guidelines |
| Hexavalent Chromium, Cr (VI) | Yes | Direct push, Hollow stem auger, Bucket auger, Hand auger, Power auger, Spiral auger with conditions in regulations | Analytical method: EPA 7199 Preparation method: EPA 3060 | 100-250 ml glass jar with Teflon-lined cap with no air space or Inert liner sealed on both ends or Polyethylene container | Cool, 4°C±2 | 30 days to extraction 7 days from extraction | All sample containers must be demonstrated to be free of contamination at or below the reporting limit. Plastic and glass containers are both suitable. Must be consistent with EPA metal guidelines |
| Explosives **See analytical list below (NO.5) | Yes | Direct Push, Hollow Stem auger, hand auger, power auger, spiral auger w/SPT | Analytical method: EPA 8330B | Amber Jar 100-250 ml, Teflon Capped Jar or inert liner seal on both ends and keep out of sun; | Cool, 4°C±2; Avoid exposure to light | 14 days to extraction; 40 days from extraction to analysis |  |
| Perchlorate | Yes | Direct Push, Hollow Stem auger, hand auger, power auger, spiral auger w/SPT | Analytical method: EPA Method 6850 or EPA Method 6860 | 100-250 ml glass jar with Teflon-lined cap with no air space | Cool, 4°C±2; Store sample or extract with headspace to avoid anerobic biodegradation | Extraction and analyze within 28 days of collection |  |

|  |  | ## TABLE 2 ## SAMPLING QUALITY CONTROL (6) |  |
| --- | --- | --- | --- |
| QC Sample | Required Frequency | Applicable Analytical Parameters | Required Sampling Containers/Preservation/Comments |
| Field Duplicates | 1 per every 20 samples or 1 if less than 10 samples. | All | The analysis lab will split the sample after homogenization, and conduct the analysis for both samples (Not applicable for VOC and TPHs). See TO-15 guidelines for required quality control sampling. |
| Split Samples | 10% of the samples for each analytical parameter, when there are at least 10 samples | All | The split will be done in the field and samples will be sent to two different accredited labs that will conduct the analysis in the same analytical methods. Split sample is not required for VOC and TPH-GRO. See TO-15 guidelines for required quality control sampling. Ex: Total samples = 100 (35 TPH only; 60 TPH + metal; 5 metals + SVOC). You must split 9 TPH (from the 95 TPH samples), 6 metals, 0 SVOC) |
| Soil Background Samples | 1 per Site | All | It is acceptable to take the background samples inside the contaminated site, but must be taken in an area that is far from the contaminated activities. See TO-15 guidelines for required quality control sampling. |
| Air Field Blank (only if required by IMOEP) | 1 per every 20 samples or 1 per site if less than 10 samples | All | It is important to take samples from workplace air, from representative points in the site. VOC & TPH-GRO: 22-mL clear glass vial should contain 10 mL matrix modifying solution (4) with PTFE-lined septum and crimp-cap or screw-caps. SVOC, TPH-DRO, TPH-ORO: 1-Liter amber glass bottle should contain water with Teflon-lined cap Metals: 500 mL polyethylene or glass bottle preserved with HNO3 to pH <2 TO-15 - See guidelines for required sampling quality control. |

|  |  | TABLE 2 SAMPLING QUALITY CONTROL(6) |  |
| --- | --- | --- | --- |
| QC Sample | Recommended/Required Frequency | Applicable Analytical parameter (from table 1( | Required Sampling Containers/Preservation/Comments |
| Equipment Blank (analysis required only when reusable equipment is used and only if required by IMOEP) | 1 per 20 samples for non-disposable sampling equipment | All | If the accredited worker who takes the samples conducts a validation test for the equipment cleaning process, he is exempt from taking samples for the equipment blank. (Validation test will be valid for 2 years). VOC & TPH-GRO: 22-mL clear glass vial with PTFE-lined septum and crimp-cap or screw-caps SVOC, TPH-DRO, TPH-ORO: 1-Liter amber glass vial with Teflon-lined cap Metals: 500 mL polyethylene or glass bottle preserved with HNO3 to pH <2 |
| Trip Blank (only if required by IMOEP) | 1 per cooler | VOC | Trip sample should be held in cooler with the samples. 22-mL clear glass vial should contain 10 mL matrix modifying solution (4) and about 2-grams clean soil with PTFE-lined septum and crimp-cap or screw-caps. |
| Cooler Temperature Blank (only if required by IMOEP) | 1 per cooler | All | Need ice volume of not less than 20% of cooler space volume. The ice will be held in a waterproof container. 125 ml glass or polyethylene container with water TO-15 : not required for canisters. |
| Additional contaminate that exists in the sample and is not included in the lab list | 10% of the samples (low-concentration and high- concentration) | VOC SVOC | If the difference between the total concentration of the compounds in the reported list and the total VOC/SVOCs is more than the order of magnitude, the lab should note the highest concentration that is not included in the lab’s standard list (see standard list on next page). That concentration must be clearly noted on the lab report, and must be considered in the site investigation report. |

- Homogenization of samples submitted in a liner must be performed by the analytical laboratory.

- Per the Israel regulations. NOTE: Scoop/trowel, hand auger, and bucket auger not allowed for VOCs and TPH-GRO; power auger and backhoe loader not allowed for VOCs, TPH-GRO, TPH-DRO and TPH-ORO.

- Sampling laboratory must inform analytical laboratory of need for low-concentration (PID<20 ppm) VOC analyses, high-concentration (PID>20 ppm) VOC analyses, or both (will be dependent on whether concentrations of VOCs are unknown and/or what regulatory cleanup levels are being used at the site).

- Matrix Modifying Solution: Based on EPA 5021 June 2003, part 7.7 (180 gr NaCl to 500 ml water).

- Preservation matrix must be prepared by analytical Laboratory and supplied within vials and Jars.

- Quality Control (QC) samples referenced in Israeli Regulations (Reference 122-09) included in table with “Frequencies Recommended/Required” in Regulations; actual number of QC samples will be dependent on project objectives and may vary from site to site

## Analytical List 1 – Standard list for VOC (GC-MS-HS)

## Method EPA 8260

***Limit of Report (LOR) must be lower than VSL values***

|  | compound | CAS No. |
| --- | --- | --- |
| 1 | Acetone | 67-64-1 |
| 2 | Benzene | 71-43-2 |
| 3 | Bromobenzene | 108-86-1 |
| 4 | Bromochloromethane | 74-97-5 |
| 5 | Bromodichloromethane | 75-27-4 |
| 6 | Bromoform | 75-25-2 |
| 7 | Bromomethane | 74-83-9 |
| 8 | Butylbenzene, n- | 104-51-8 |
| 9 | Butylbenzene, sec- | 135-98-8 |
| 10 | Butylbenzene, tert- | 98-06-6 |
| 11 | Carbon Tetrachloride | 56-23-5 |
| 12 | Chlorobenzene | 108-90-7 |
| 13 | Chloroform | 67-66-3 |
| 14 | Chloromethane | 74-87-3 |
| 15 | Chlorotoluene, o- | 95-49-8 |
| 16 | Chlorotoluene, p- | 106-43-4 |
| 17 | Cumene (Isopropylbenzene) | 98-82-8 |
| 18 | Dibromo-3-chloropropane, 1,2- | 96-12-8 |
| 19 | Dibromochloromethane | 124-48-1 |
| 20 | Dibromoethane, 1,2- (EDB) | 106-93-4 |
| 21 | Dibromomethane (Methylene Bromide) | 74-95-3 |
| 22 | Dichlorobenzene, 1,2- | 95-50-1 |
| 23 | Dichlorobenzene, 1,4- | 106-46-7 |
| 24 | Dichlorodifluoromethane | 75-71-8 |
| 25 | Dichloroethane, 1,1- | 75-34-3 |
| 26 | Dichloroethane, 1,2- (EDC) | 107-06-2 |
| 27 | Dichloroethylene, 1,1- | 75-35-4 |
| 28 | Dichloroethylene, 1,2-cis- | 156-59-2 |
| 29 | Dichloroethylene, 1,2-trans- | 156-60-5 |
| 30 | Dichloropropane, 1,2- | 78-87-5 |
| 31 | Dichloropropane, 1,3- | 142-28-9 |
| 32 | Dioxane 1,4 | 123-91-1 |
| 33 | Ethyl Chloride (Chloroethane) | 75-00-3 |
| 34 | Ethylbenzene | 100-41-4 |
| 35 | Hexachlorobutadiene | 87-68-3 |
| 36 | Hexane, N- | 110-54-3 |
| 37 | Methyl Ethyl Ketone – MEK (2-Butanone) | 78-93-3 |

## Standard list for VOC (GC-MS-HS) (Analytical List 1 – continue)

## Method EPA 8260

***Limit of Report (LOR) must be lower than VSL values***

|  | compound | CAS No. |
| --- | --- | --- |
| 38 | Methyl Isobutyl Ketone – MIBK (4-methyl-2-pentanone) | 108-10-1 |
| 39 | Methyl tert-Butyl Ether (MTBE) | 1634-04-4 |
| 40 | Methylene Chloride (dichloromethane) | 75-09-2 |
| 41 | Naphthalene | 91-20-3 |
| 42 | Propyl benzene | 103-65-1 |
| 43 | Styrene | 100-42-5 |
| 44 | Tetrachloroethane, 1,1,1,2- | 630-20-6 |
| 45 | Tetrachloroethane, 1,1,2,2- | 79-34-5 |
| 46 | Tetrachloroethylene (PCE) | 127-18-4 |
| 47 | Toluene | 108-88-3 |
| 48 | Trichlorobenzene, 1,2,3- | 87-61-6 |
| 49 | Trichlorobenzene, 1,2,4- | 120-82-1 |
| 50 | Trichloroethane, 1,1,1- | 71-55-6 |
| 51 | Trichloroethane, 1,1,2- | 79-00-5 |
| 52 | Trichloroethylene (TCE) | 79-01-6 |
| 53 | Trichloropropane, 1,2,3- | 96-18-4 |
| 54 | Trimethylbenzene, 1,2,4- | 95-63-6 |
| 55 | Trimethylbenzene, 1,3,5- | 108-67-8 |
| 56 | Vinyl Chloride | 75-01-4 |
| 57 | Xylene (m-, o-, p-) | (108-38-3, 95-47-6, 106-42-3) |
| 58 | Xylenes | 1330-20-7 |

## Analytical List 2 – Standard list for VOCs (Active Soil Gas)

## Method EPA TO-15

***Limit of Report (LOR) must be lower than VSL values***

|  | compound | CAS No. |
| --- | --- | --- |
| 1 | Vinyl Acetate | 108-05-4 |
| 2 | 2-Propanol (Isopropyl Alcohol) | 67-63-0 |
| 3 | 2-Butanone (MEK) | 78-93-3 |
| 4 | Acrolein | 107-02-8 |
| 5 | Benzyl Chloride | 100-44-7 |
| 6 | Carbon Disulfide | 75-15-0 |
| 7 | Cyclohexane | 110-82-7 |
| 8 | Ethyl Acetate | 141-78-6 |
| 9 | Methyl Methacrylate | 80-62-6 |
| 10 | 1,1,1-Trichloroethane | 71-55-6 |
| 11 | 1,2,4-Trichlorobenzene | 120-82-1 |
| 12 | 1,2-Dichlorobenzene | 95-50-1 |
| 13 | Methyl tert-Butyl Ether | 1634-04-4 |
| 14 | 1,1,2,2-Tetrachloroethane | 79-34-5 |
| 15 | 1,1,2-Trichloroethane | 79-00-5 |
| 16 | 1,1-Dichloroethane | 75-34-3 |
| 17 | 1,1-Dichloroethene | 75-35-4 |
| 18 | 1,2,4-Trimethylbenzene | 95-63-6 |
| 19 | 1,2-Dibromo-3-chloropropane | 96-12-8 |
| 20 | 1,2-Dibromoethane | 106-93-4 |
| 21 | 1,2-Dichloroethane | 107-06-2 |
| 22 | 1,2-Dichloropropane | 78-87-5 |
| 23 | 1,3,5-Trimethylbenzene | 108-67-8 |
| 24 | 1,3-Butadiene | 106-99-0 |
| 25 | 1,4-Dichlorobenzene | 106-46-7 |
| 26 | 1,4-Dioxane | 123-91-1 |
| 27 | 2-Hexanone | 591-78-6 |
| 28 | 3-Chloro-1-propene (Allyl Chloride) | 107-05-1 |
| 29 | 4-Methyl-2-pentanone | 108-10-1 |
| 30 | Acetonitrile | 75-05-8 |
| 31 | Acrylonitrile | 107-13-1 |
| 32 | Benzene | 71-43-2 |
| 33 | Bromodichloromethane | 75-27-4 |
| 34 | Bromoform | 75-25-2 |
| 35 | Bromomethane | 74-83-9 |
| 36 | Carbon Tetrachloride | 56-23-5 |
| 37 | Chlorobenzene | 108-90-7 |
| 38 | Chloroethane | 75-00-3 |
| 39 | Chloroform | 67-66-3 |
| 40 | Chloromethane | 74-87-3 |
| 41 | cis-1,2-Dichloroethene | 156-59-2 |
| 42 | Cumene | 98-82-8 |
| 43 | Dichlorodifluoromethane (CFC 12) | 75-71-8 |
| 44 | Ethylbenzene | 100-41-4 |
| 45 | Hexachlorobutadiene | 87-68-3 |
| 46 | Methylene Chloride | 75-09-2 |
| 47 | Naphthalene | 91-20-3 |
| 48 | n-Hexane | 110-54-3 |
| 49 | n-Nonane | 111-84-2 |
| 50 | n-Propylbenzene | 103-65-1 |
| 51 | o-Xylene | 95-47-6 |
| 52 | p+m – Xylene | 108-38-3, 106-42-3 |
| 53 | Propene | 115-07-1 |
| 54 | Styrene | 100-42-5 |
| 55 | Tetrachloroethene | 127-18-4 |
| 56 | Tetrahydrofuran (THF) | 109-99-9 |
| 57 | Toluene | 108-88-3 |
| 58 | Trichloroethene | 79-01-6 |
| 59 | Trichlorotrifluoroethane | 76-13-1 |
| 60 | Vinyl Chloride | 75-01-4 |

## Analytical List 3 – Standard list for SVOCs (GC-MS-HS)

## Method EPA 8270

***Limit of Report (LOR) must be lower than VSL values***

|  | compound | CAS No. |
| --- | --- | --- |
| 1 | Acenaphthene | 83-32-9 |
| 2 | Acetophenone | 98-86-2 |
| 3 | Anthracene | 120-12-7 |
| 4 | Benz[a]anthracene | 56-55-3 |
| 5 | Benzo(a)pyrene | 50-32-8 |
| 6 | Benzo(b)fluoranthene | 205-99-2 |
| 7 | Benzo(k)fluoranthene | 207-08-9 |
| 8 | Benzyl Alcohol | 100-51-6 |
| 9 | Biphenyl, 1,1- | 92-52-4 |
| 10 | Bis(2-chloroethoxy)methane | 111-91-1 |
| 11 | Bis(2-ethylhexyl)phthalate | 117-81-7 |
| 12 | Caprolactam | 105-60-2 |
| 13 | Chloronaphthalene, Beta- | 91-58-7 |
| 14 | Chlorophenol, 2- | 95-57-8 |
| 15 | Chrysene | 218-01-9 |
| 16 | Dibenz[a,h]anthracene | 53-70-3 |
| 17 | Dibutyl Phthalate | 84-74-2 |
| 18 | Dichlorophenol, 2,4- | 120-83-2 |
| 19 | Diethyl Phthalate | 84-66-2 |
| 20 | Dimethylphenol, 2,4- | 105-67-9 |
| 21 | Dinitrophenol, 2,4- | 51-28-5 |
| 22 | Dinoseb | 88-85-7 |
| 23 | Diphenylamine | 122-39-4 |
| 4 | Fluoranthene | 206-44-0 |
| 25 | Fluorene | 86-73-7 |
| 26 | Hexachlorocyclopentadiene | 77-47-4 |
| 27 | Indeno[1,2,3-cd]pyrene | 193-39-5 |
| 28 | Isophorone | 78-59-1 |
| 29 | Methylnaphthalene, 2- | 91-57-6 |
| 31 | Octyl Phthalate, di-N- | 117-84-0 |
| 32 | Pentachlorophenol | 87-86-5 |
| 33 | Phenol | 108-95-2 |
| 34 | Pyrene | 129-00-0 |
| 35 | Trichlorophenol, 2,4,5- | 95-95-4 |
| 36 | Trichlorophenol, 2,4,6- | 88-06-2 |

## Analytical List 4 – Standard list for metals (ICP-MS)

## Method EPA 6010D/6020A

***Limit of Report (LOR) must be lower than VSL values***

|  | compound | CAS No. |
| --- | --- | --- |
| 1 | Aluminum | 7429-90-5 |
| 2 | Antimony | 7440-36-0 |
| 3 | Arsenic | 7440-38-2 |
| 4 | Barium | 7440-39-3 |
| 5 | Beryllium and compounds | 7440-41-7 |
| 6 | Cadmium | 7440-43-9 |
| 7 | Chromium, Total | 7440-47-3 |
| 8 | Cobalt | 7440-48-4 |
| 9 | Copper | 7440-50-8 |
| 10 | Iron | 7439-89-6 |
| 11 | Lead | 7439-92-1 |
| 12 | Lithium | 7439-93-2 |
| 13 | Manganese | 7439-96-5 |
| 14 | Mercury | 7439-97-6 |
| 15 | Molybdenum | 7439-98-7 |
| 16 | Nickel | 7440-02-0 |
| 17 | Selenium | 7782-49-2 |
| 18 | Silver | 7440-22-4 |
| 19 | Thallium | 7440-28-0 |
| 20 | Vanadium | 7440-62-2 |
| 21 | Zinc | 7440-66-6 |

## Analytical List 5 – Standard list for Explosives

## Method EPA 8330

|  | Acronym | Compound Name | CAS |
| --- | --- | --- | --- |
| 1 | TNT | 2,4,6-trinitrotoluene | 118-96-7 |
| 2 | TNB | 1,3,5-trnitrobenzene (TNT Breakdown product) | 99-35-4 |
| 3 | 2,4-DNT | 2,4-dinitrotoluene | 121-14-2 |
| 4 | 2,6-DNT | 2,6-dinitrotoluene | 606-20-2 |
| 5 | Tertryl | Methyl-2,4,6-trinitrophenylnitramine | 479-45-8 |
| 6 | 2-AmDNT | 2-amino-4,6-dinitrotoluene (TNT Breakdown product) | 35572-78-2 |
| 7 | 4-AMDNT | 4-amino-2,6-dinitrotoluene (TNT Breakdown product) | 19406-51-0 |
| 8 | 2-NT | 2-Nitrotoluene or m-Nitrotoluene | 99-08-1 |
| 9 | 3-NT | 3-Nitrotoluene or o-Nitrotoluene | 88-72-2 |
| 10 | 4-NT | 4-Nitrotoluene pr p-Nitrotoluene | 99-99-0 |
| 11 | NB | Nitrobenzene | 98-95-3 |
| 12 | 1,3-DNB | 1,3-Dinitrobenzene | 99-65-0 |
| 13 | RDX | Hexahydor-1,3,5-trinitro-1,3,5-triazine | 121-82-4 |
| 14 | HMX | Octahydro-1,3,5-7-tetranitro-1,3,5,7-tetrazoncine | 2691-41-0 |
| 15 | NG | Nitroglycerin | 55-63-0 |
| 16 | 3,5 DNA | 3,5 dinitroaniline (TNT Breakdown product) | 618-87-1 |
| 17 | PETN | Pentaerythritol tetranitrate | 78-11-5 |

***Limit of Report (LOR) must be lower than VSL values***

## הנחיות לשינוע דגימות ממעבדת הדיגום בישראל אל מעבדה אנליטית בחו"ל

תהליך השינוע כולו הינו באחריות המעבדה הדוגמת, והוא יפורט בנהלי עבודה אשר יהיו חלק מההסמכה של הרשות להסמכת מעבדות עבור פעילות המעבדה. נהלי העבודה של המעבדה הדוגמת יכללו התייחסות לכל השלבים מרגע נטילת הדגימה בישראל ועד להגעתה אל המעבדה האנליטית בחו"ל המוסמכת לשיטה המצוינת בטבלה 1 במסמך זה, לרבות הדרישות הבאות:

- אחסון

  - יש לתעד עבור כל אריזה ועבור כל דגימה את תנאי האחסון מרגע הדיגום ועד הגעה לאנליזה במעבדה האנליטית לרבות מיקום, האם נשמרה בקירור\הקפאה, משך האחסון בכל מקום וכד'.

  - אחסון במקרר או במקפיא יהיה בכפוף להסמכה של הציוד ושל צורת הכיול שלו ובמעבדה מוסמכת ע"י הרשות להסמכת מעבדות. הקפאה של דגימות עם תכולת רטיבות גבוהה תהיה בהדרגתיות (קודם קירור ולאחר מכן הקפאה) וזאת בכדי למנוע התפשטות מהירה של החומר וסדיקת הכלי.

  - מיקום אחסון הדגימות במעבדה הדוגמת יוגדר על פי נהלי העבודה הפנימיים ויתבצע במקום נעול, או לחלופין תחת השגחה רציפה.

- אריזה

האריזה תתבצע באופן שימנע כל פגיעה בדגימות, לרבות –

  - אריזה חיצונית קשיחה למניעת מעיכה (קרטון, פלסטיק וכד').

  - סימון חיצוני על גבי האריזה של מטען שביר בעברית ובאנגלית.

  - אריזת כל כלי זכוכית (צנצנת, ווייל וכד') בעטיפה נפרדת המונעת פגיעה וחיכוך עם כלי זכוכית אחרים.

  - השלמת נפח באריזה באמצעות חומרי מילוי ייעודיים (ספוגים, קלקרים, "פצפצים" וכד') כך שלא תתאפשר תזוזה של הדגימות.

  - הגנה על מסמכים מצורפים מפני רטיבות.

  - יש לדרוש מהמעבדה האנליטית לדווח בכתב על כל סימן חיצוני או פנימי באריזות המעידים על חשש מפגיעה בדגימות – לדוגמא שברי זכוכית באריזה, רטיבות, מעיכה, סימנים של פתיחת האריזה וכד'. במידה ויש חשש לפגיעה או שבוודאות ידוע על פגיעה, יש לדווח על כך בדו"ח ממצאי המעבדה ולפסול את הדגימה שאינה עומדת בסטנדרטים הנדרשים.

- תיעוד

אל האריזה יצורפו מסמכי תיעוד בשפה האנגלית אשר יאפשרו בקרה של המעבדה האנליטית על כך שכלל הדגימות שנשלחו הגיעו אל היעד בשלמותן, לרבות –

  - רישום של כמות הדגימות בכל אריזה, סוג המיכל בהן הן נמצאות (בקבוקים, צנצנות, קניסטרים וכד') ונפח המיכל.

  - במידה ונשלחות דגימות ממספר פרויקטים שונים באותה אריזה, יש לדאוג לזיהוי חד-חד ערכי עבור כל דגימה, על מנת למנוע בלבול כתוצאה משמות חוזרים.

  - רישום של מספר האריזה מתוך כלל האריזות במשלוח ("אריזה מס' X מתוך Y").

  - רישום משקל אריזה כולל בעת האריזה.

  - שימוש בחותם הסוגר את האריזה כך שניתן יהיה להבחין האם היא נפתחה במהלך השינוע עד להגעה למעבדה.

- טמפרטורה

טמפרטורת הדגימות תשמר לאורך כל השינוע אל המעבדה האנליטית כך שתעמוד בדרישות בנושא זה על פי ההסמכה הרלוונטית לשיטת האנליזה הנדרשת.

  - אריזת דגימות הדורשות שמירה על טמפרטורה נמוכה לאורך השינוע תהיה באריזה פנימית מבודדת חום, לדוגמא צידנית פלסטיק, קלקר וכד' וזאת, בנוסף לאריזה הקשיחה החיצונית.

  - הכנסת קרח לאריזה תהיה רק בכלים אטומים ("קרחומים" או שקיות אטומות) אשר ימנעו נזילת מים עם הפשרת הקרח.

  - לכל משלוח הנדרש בקירור יצורף מכשיר מדידה רציף לטמפרטורה (Data Logger), עם יכולת שמירה של הנתונים ושליפתם על ידי המעבדה האנליטית. נתונים אלו יוצגו בדו"ח ממצאי המעבדה. יש להקפיד על רישום מדויק של זמן שליחת הדגימה וזמן קבלת הדגימה במעבדה, ולהקפיד על הפעלה והפסקה של המדידה במכשיר בהתאם לזמנים אלו.

- שילוח

שילוח האריזות מחייב מעבר רציף ואמין בין הגורמים השונים עד להגעתן למעבדה, תוך בקרה ותיעוד, לרבות הדרישות הבאות:

  - שילוח יבוצע רק באמצעות חברות המספקות מספר משלוח המאפשר מעקב רציף אחר מיקום ומצב כל אריזה.

  - יעשה שימוש בטופס שרשרת משמורת בשפה האנגלית עם תיעוד מלא של תאריך, שעה, מיקום ומצב האריזה על ידי כל גורם עד להגעה למעבדה. המעבדה האנליטית נדרשת לחתום על הטופס עם קבלת הדגימות.

  - אל המשלוח יש לצרף בצורה בולטת את כל המסמכים, האישורים וההצהרות הנדרשות עבור שינוע הדגימות דרך נמלי תעופה, מעברי גבול, מכסים ובדיקות הרשויות השונות, כך שלא יווצרו עיכובים או צורך בפתיחה של האריזה (לדוגמא – אישורים עבור שינוע חומ"ס, קרקעות, נוזלים, נדיפים, נושאי בטיחות, מכס, הגבלות חקלאיות וכד')

  - המשלוח יעשה בצירוף כתובת למשלוח חוזר של מעבדת הדיגום, בצירוף פרטי קשר מלאים של השולח.

תיעוד המשלוח יצורף לדוח המעבדה ולדוח התוצאות הסופי שמוגש למשרד להגנת הסביבה.